FEATURE LOGIC EXPLANATION

1. Overview of Implemented Features

The proposed system simulates an Advanced Driver Assistance System (ADAS) focused on preventing frontal collisions using Time-to-Collision (TTC) based decision-making.

The system continuously monitors the distance between the vehicle and a stationary obstacle and dynamically evaluates the risk level.

Key Features:

- Real-time distance monitoring

- Continuous TTC calculation

- Multi-level safety classification

- Visual alert system for driver awareness

- Automatic emergency braking

2. Time-to-Collision (TTC) Calculation

Time-to-Collision (TTC) is a critical parameter used to estimate the time remaining before a collision occurs if the current speed is maintained.

Formula:

TTC=Distance between vehicle and obstacleVehicle speed

Implementation:

- Distance is calculated as:

Distance=Obstacle Position-Vehicle Position

- TTC is updated in real-time at every simulation step.

Interpretation:

- Higher TTC → Safer condition

- Lower TTC → Higher collision risk

3 Threshold-Based Decision System

The system uses predefined TTC thresholds to classify driving conditions into three safety zones:

| TTC Range | Zone | System Action |
| --- | --- | --- |
| TTC > 10 sec | Safe Zone | Normal driving |
| 5 < TTC ≤ 10 | Warning Zone | Driver alert generated |
| TTC ≤ 5 sec | Danger Zone | Automatic braking activated |

Purpose of Thresholds:

- 10 seconds provides sufficient reaction time for the driver

- 5 seconds represents a critical limit requiring immediate intervention

4. Arbitration Logic (Decision Priority)

The system follows a priority-based decision-making mechanism to ensure safety.

Logic Implementation:

- The system evaluates TTC conditions sequentially.

- The most critical condition is given highest priority.

🔺 Priority Order:

- 🔴 Automatic Braking (Highest Priority)

- 🟡 Warning Alert

- 🟢 Safe Condition

Explanation:

Even if multiple conditions appear valid, the system always selects the most critical action.

Example:

- If TTC = 4 seconds

  - Warning condition is true

  - Danger condition is also true

System chooses Automatic Braking because it has higher priority.

5. Automatic Braking Mechanism

When TTC falls below the critical threshold (≤ 5 seconds), the system activates automatic braking.

Implementation:

- Vehicle speed is set to zero:

speed = 0;

Effects:

- Vehicle stops immediately

- Distance stops decreasing

- Collision is prevented

6. System Behaviour Summary

The system operates in a continuous loop and responds dynamically:

- Vehicle moves toward obstacle

- Distance decreases over time

- TTC is continuously recalculated

- Warning is issued when TTC < 10

- Automatic braking is applied when TTC ≤ 5

- Vehicle comes to a complete stop

7. Visual Feedback Integration

To enhance user understanding, the system provides real-time visual feedback:

- Color-coded vehicle states:

  - 🟢 Green → Safe

  - 🟡 Yellow → Warning

  - 🔴 Red → Danger

- Alert messages displayed on screen

- Graph showing TTC and distance trends